Data Sheet, Rev. 1. 1 , December 2004

# BTS 7960

High Current PN Half Bridge NovalithIC TM

43 A, 7 m + 9 m

Automotive Power

N e v e r	s t o p	t h i n k i n g .

Product Summary	2

Basic Features	2

- Overview	3

  - Block Diagram	3

  - Terms	4

- Pin Configuration	5

  - Pin Assignment	5

  - Pin Definitions and Functions	5

- Maximum Ratings	6

- Block Description and Characteristics	7

  - Supply Characteristics	7

  - Power Stages	8

    - Power Stages - Static Characteristics	9

    - Switching Times	10

    - Power Stages - Dynamic Characteristics	11

  - Protection Functions	13

    - Overvoltage Lock Out	13

    - Undervoltage Shut Down	13

    - Overtemperature Protection	13

    - Current Limitation	13

    - Short Circuit Protection	15

    - Electrical Characteristics - Protection Functions	16

  - Control and Diagnostics	17

    - Input Circuit	17

    - Dead Time Generation	17

    - Adjustable Slew Rate	17

    - Status Flag Diagnosis With Current Sense Capability	17

    - Truth Table	19

    - Electrical Characteristics - Control and Diagnostics	20

- Thermal Characteristics	21

- Application	22

  - Application Example	22

  - Layout Considerations	22

- Package Outlines P-TO-263-7	23

- Package Outlines P-TO-220-7	24

- Revision History	25

### High Current PN Half Bridge NovalithICTM

## BTS 7960B BTS 7960P

BTS 7960B P-TO-263-7

Product Summary

BTS 7960P P-TO-220-7

The BTS 7960 is a fully integrated high current half bridge for motor drive applications. It is part of the NovalithICTM family containing one p-channel highside MOSFET and one n-channel lowside MOSFET with an integrated driver IC in one package. Due to the p-channel highside switch the need for a charge pump is eliminated thus minimizing EMI. Interfacing to a microcontroller is made easy by the integrated driver IC which features logic level inputs, diagnosis with current sense, slew rate adjustment, dead time generation and protection against overtemperature, overvoltage, undervoltage, overcurrent and short circuit.

The BTS 7960 provides a cost optimized solution for protected high current PWM motor drives with very low board space consumption.

### Basic Features

- Path resistance of typ. 16 m @ 25 C

- Low quiescent current of typ. 7 A @ 25 C

- PWM capability of up to 25 kHz combined with active freewheeling

- Switched mode current limitation for reduced power dissipation in overcurrent

- Current limitation level of 43 A typ.

- Status flag diagnosis with current sense capability

- Overtemperature shut down with latch behaviour

- Overvoltage lock out

- Undervoltage shut down

- Driver circuit with logic level inputs

- Adjustable slew rates for optimized EMI

| Type | Ordering Code | Package |
| --- | --- | --- |
| BTS 7960B | Q67060-S6160 | P-TO-263-7 |
| BTS 7960P | on request | P-TO-220-7 |

Overview

# Overview

The BTS 7960 is part of the NovalithIC family containing three separate chips in one package: One p-channel highside MOSFET and one n-channel lowside MOSFET together with a driver IC, forming a fully integrated high current half-bridge. All three chips are mounted on one common leadframe, using the chip on chip and chip by chip technology. The power switches utilize vertical MOS technologies to ensure optimum on state resistance. Due to the p-channel highside switch the need for a charge pump is eliminated thus minimizing EMI. Interfacing to a microcontroller is made easy by the integrated driver IC which features logic level inputs, diagnosis with current sense, slew rate adjustment, dead time generation and protection against overtemperature, overvoltage, undervoltage, overcurrent and short circuit. The BTS 7960 can be combined with other BTS 7960 to form H-bridge and 3-phase drive configurations.

### Block Diagram

GND

IS

INH

SR

OUT

IN

VS

LS base-chip

Top-chip

Gate Driver Dead Time Gen. Slew Rate Adj. UV Shut Down OV Lock Out OT Shut Down Current Lim.

Diagnosis

Current Sense

HS base-chip

BTS 7960

Figure 1    Block Diagram

Overview

### Terms

Following figure shows the terms used in this data sheet.

IINH

IGND, ID(LS)

VIS

IIS

V SR

V OUT

IOUT , I L

VSD (L S)

ISR

VINH

VS

V IN

IIN

V D S(H S)

IVS , -ID(HS)

VVS ,VS

GND

IS

SR

OUT

BTS 7960

INH

IN

Figure 2	Terms

Pin Configuration

# Pin Configuration

### Pin Assignment

BTS 7960B

P-TO-263-7

BTS 7960P P-TO-220-7

8

8

1 2 3 4 5 6 7

1	3	5

2	4	6

7

Figure 3	Pin Assignment BTS 7960B and BTS 7960P (top view)

### Pin Definitions and Functions

| Pin | Symbol | I/O | Function |
| --- | --- | --- | --- |
| 1 | GND | - | Ground |
| 2 | IN | I | Input Defines whether high- or lowside switch is activated |
| 3 | INH | I | Inhibit When set to low device goes in sleep mode |
| 4,8 | OUT | O | Power output of the bridge |
| 5 | SR | I | Slew Rate The slew rate of the power switches can be adjusted by connecting a resistor between SR and GND |
| 6 | IS | O | Current Sense and Diagnosis |
| 7 | VS | - | Supply |

Bold type: pin needs power wiring

Maximum Ratings

# Maximum Ratings

-40 C < Tj < 150 C (unless otherwise specified)

| Pos | Parameter | Symbol | Limits |  | Unit | Test Condition |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | min | max |  |  |

Electrical Maximum Ratings

| 3.0.1 | Supply voltage | VVS | -0.3 | 45 | V |  |
| --- | --- | --- | --- | --- | --- | --- |
| 3.0.2 | Logic Input Voltage | VIN VINH | -0.3 | 5.3 | V |  |
| 3.0.3 | HS/LS continuous drain current | ID(HS) ID(LS) | -40 | 401) | A | TC < 85°C switch active |
| 3.0.4 | HS pulsed drain current | ID(HS) | -60 | 601) | A | TC < 85°C tpulse = 10ms |
| 3.0.5 | LS pulsed drain current | ID(LS) | -60 | 601) | A |  |
| 3.0.6 | Voltage at SR pin | VSR | -0.3 | 1.0 | V |  |
| 3.0.7 | Voltage between VS and IS pin | VVS -VIS | -0.3 | 45 | V |  |
| 3.0.8 | Voltage at IS pin | VIS | -20 | 45 | V |  |

Thermal Maximum Ratings

| 3.0.9 | Junction temperature | Tj | -40 | 150 | C |  |
| --- | --- | --- | --- | --- | --- | --- |
| 3.0.10 | Storage temperature | Tstg | -55 | 150 | C |  |

ESD Susceptibility

| 3.0.11 | ESD susceptibility HBM | VESD | -2 -6 | 2 6 | kV | according to EIA/ JESD 22-A 114B |
| --- | --- | --- | --- | --- | --- | --- |
|  | IN, INH, SR, IS OUT, GND, VS |  |  |  |  |  |

1)  Maximum reachable current may be smaller depending on current limitation level

Note: Maximum ratings are absolute ratings; exceeding any one of these values may cause irreversible damage to the device. Exposure to maximum rating conditions for extended periods of time may affect device reliability

Block Description and Characteristics

# Block Description and Characteristics

### Supply Characteristics

– 40 C < Tj < 150 C, 8 V < VS < 18 V, IL = 0A (unless otherwise specified)

| Pos. | Parameter | Symbol | Limit Values |  |  | Unit | Test Conditions |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | min. | typ. | max. |  |  |

General

| 4.1.1 | Operating Voltage | VS | 5.5 | – | 27.5 | V |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 4.1.2 | Supply Current | IVS(on) | – | 2 | 3 | mA | VINH = 5 V VIN = 0 V or 5 V RSR=0  DC-mode normal operation (no fault condition) |
| 4.1.3 | Quiescent Current | IVS(off) | – | 7 | 15 | µA | VINH = 0 V VIN = 0 V or 5 V Tj <85 C |
|  |  |  | – | – | 65 | µA | VINH = 0 V VIN = 0 V or 5 V |

Block Description and Characteristics

### Power Stages

The power stages of the BTS 7960 consist of a p-channel vertical DMOS transistor for the high side switch and a n-channel vertical DMOS transistor for the low side switch. All protection and diagnostic functions are located in a separate top chip. Both switches can be operated up to 25 kHz, allowing active freewheeling and thus minimizing power dissipation in the forward operation of the integrated diodes.

The on state resistance RON is dependent on the supply voltage VS as well as on the junction temperature Tj . The typical on state resistance characteristics are shown in Figure 4.

25

V 28

4	8	12	16

5

5

j

Tj = -40°C

Tj = 25°C

T = -40°C

j

T = 25°C

8

10

12

15

m

25

20	24

VS

m

20

20

RON(HS)

RON(LS)

Low Side Switch

15

Tj = 150°C

16	20	24  V 28

VS

Tj = 150°C

10

High Side Switch

4

Figure 4    Typical On State Resistance vs. Supply Voltage

Block Description and Characteristics

### Power Stages - Static Characteristics

– 40 C < Tj < 150 C, 8 V < VS < 18 V (unless otherwise specified)

| Pos. | Parameter | Symbol | Limit Values |  |  | Unit | Test Conditions |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | min. | typ. | max. |  |  |

High Side Switch - Static Characteristics

| 4.2.1 | On state high side resistance | RON(HS) | – – | 7 10 | 9 12.5 | m | IOUT = 9 A VS= 13.5 V Tj = 25 C Tj = 150 C |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 4.2.2 | Leakage current high side | IL(LKHS) | – | – | 1 | µA | VINH = 0 V VOUT = 0 V Tj < 85 C |
|  |  |  | – | – | 50 | µA | VINH = 0 V VOUT = 0 V Tj = 150 C |
| 4.2.3 | Reverse diode forward-voltage high side 1) | VDS(HS) | – – – | 0.9 0.8 0.6 | 1.5 1.1 0.8 | V | IOUT = -9 A Tj = -40 C Tj = 25 C Tj = 150 C |

Low Side Switch - Static Characteristics

| 4.2.4 | On state low side resistance | RON(LS) | – – | 9 14 | 12 18 | m | IOUT = -9 A VS= 13.5V Tj = 25 C Tj = 150 C |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 4.2.5 | Leakage current low side | IL(LKLS) | – | – | 1 | µA | VINH = 0 V VOUT = VS Tj < 85 C |
|  |  |  | – | – | 15 | µA | VINH = 0 V VOUT = VS Tj = 150 C |
| 4.2.6 | Reverse diode forward-voltage low side 1) | VSD(LS) | – – – | 0.9 0.8 0.6 | 1.5 1.1 0.8 | V | IOUT = 9 A Tj = -40 C Tj = 25 C Tj = 150 C |

1)  Due to active freewheeling, diode is conducting only for a few µs, depending on RSR

Block Description and Characteristics

### Switching Times

10%

t

10%

VOUT

VOUT

VOUT

90%	90%

t

tdf (HS)	tf(HS)

tdr(HS)	tr(HS)

IN

Figure 5	Definition of switching times high side (Rload to GND)

10%

t

10%

VOUT

VOUT

90%

VOUT

90%

t

t dr(LS)	tr(LS)

t df (LS)  tf(LS)

IN

Figure 6	Definition of switching times low side (Rload to VS)

Due to the timing differences for the rising and the falling edge there will be a slight difference between the length of the input pulse and the length of the output pulse. It can be calculated using the following formulas:

      - tHS = (tdr(HS) + 0.5 tr(HS)) - (tdf(HS) + 0.5 tf(HS))

      - tLS = (tdf(LS) + 0.5 tf(LS)) - (tdr(LS) + 0.5 tr(LS)).

Block Description and Characteristics

### Power Stages - Dynamic Characteristics

-40 C < Tj < 150 C, VS = 13.5 V, Rload = 2 (unless otherwise specified)

| Pos. | Parameter | Symbol | Limit Values |  |  | Unit | Test Conditions |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | min. | typ. | max. |  |  |

HIgh Side Switch Dynamic Characteristics

| 4.2.7 | Rise-time of HS | tr(HS) | 0.5 | 1 | 1.5 | µs | RSR = 0  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | – | 2 | – |  | RSR = 5.1 k |
|  |  |  | 2.8 | 7 | 11 |  | RSR = 51 k |
| 4.2.8 | Slew rate HS on | VOUT/ tr( HS) | – – | 11 6 | – – | V/µs | RSR = 0  RSR = 5.1 k |
|  |  |  | – | 1.6 | – |  | RSR = 51 k |
| 4.2.9 | Switch on delay time HS | tdr(HS) | 1.7 | 3.1 | 4.5 | µs | RSR = 0  |
|  |  |  | – | 4.4 | – |  | RSR = 5.1 k |
|  |  |  | 5.6 | 14 | 22.4 |  | RSR = 51 k |
| 4.2.10 | Fall-time of HS | tf(HS) | 0.5 | 1 | 1.5 | µs | RSR = 0  |
|  |  |  | – | 2 | – |  | RSR = 5.1 k |
|  |  |  | 2.8 | 7 | 11 |  | RSR = 51 k |
| 4.2.11 | Slew rate HS off | -VOUT/ |  |  |  | V/µs | RSR = 0  RSR = 5.1 k RSR = 51 k |
|  |  | tf(HS) | – – | 11 6 | – – |  |  |
|  |  |  | – | 1.6 | – |  |  |
| 4.2.12 | Switch off delay time HS | tdf(HS) | 1.2 | 2.4 | 3.6 | µs | RSR = 0  |
|  |  |  | – | 3.4 | – |  | RSR = 5.1 k |
|  |  |  | 4 | 10 | 16 |  | RSR = 51 k |

Block Description and Characteristics

-40 C < Tj < 150 C, VS = 13.5 V, Rload = 2 (unless otherwise specified)

| Pos. | Parameter | Symbol | Limit Values |  |  | Unit | Test Conditions |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | min. | typ. | max. |  |  |

Low Side Switch Dynamic Characteristics

| 4.2.13 | Rise-time of LS | tr(LS) | 0.5 | 1 | 1.5 | µs | RSR = 0  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | – | 2 | – |  | RSR = 5.1 k |
|  |  |  | 2.8 | 7 | 11 |  | RSR = 51 k |
| 4.2.14 | Slew rate LS switch off | VOUT/ tr(LS) | – – | 11 6 | – – | V/µs | RSR = 0  RSR = 5.1 k |
|  |  |  | – | 1.6 | – |  | RSR = 51 k |
| 4.2.15 | Switch off delay time LS | tdr(LS) | 0.7 | 1.3 | 1.9 | µs | RSR = 0  RSR = 5.1 k RSR = 51 k |
|  |  |  | – | 2.2 | – |  |  |
|  |  |  | 2.8 | 7 | 11.2 |  |  |
| 4.2.16 | Fall-time of LS | tf(LS) | 0.5 | 1 | 1.5 | µs | RSR = 0  |
|  |  |  | – | 2 | – |  | RSR = 5.1 k |
|  |  |  | 2.8 | 7 | 11 |  | RSR = 51 k |
| 4.2.17 | Slew rate LS switch on | -VOUT/ |  |  |  | V/µs | RSR = 0  RSR = 5.1 k RSR = 51 k |
|  |  | tf(LS) | – – | 11 6 | – – |  |  |
|  |  |  | – | 1.6 | – |  |  |
| 4.2.18 | Switch on delay time LS | tdf(LS) | 2.2 | 4 | 5.8 | µs | RSR = 0  |
|  |  |  | – | 5.6 | – |  | RSR = 5.1 k |
|  |  |  | 6.4 | 16 | 25.4 |  | RSR = 51 k |

Block Description and Characteristics

### Protection Functions

The device provides integrated protection functions. These are designed to prevent IC destruction under fault conditions described in the data sheet. Fault conditions are considered as “outside” normal operating range. Protection functions are not to be used for continuous or repetitive operation, with the exception of the current limitation (Chapter 4.3.4). In a fault condition the BTS 7960 will apply the highest slew rate possible independent of the connected slew rate resistor. Overvoltage, overtemperature

and overcurrent are indicated by a fault current IIS(LIM) at the IS pin as described in the paragraph “Status Flag Diagnosis With Current Sense Capability” on Page 17 and Figure 10.

In the following the protection functions are listed in order of their priority. Overvoltage lock out overrides all other error modes.

### Overvoltage Lock Out

To assure a high immunity against overvoltages (e.g. load dump conditions) the device shuts the lowside MOSFET off and turns the highside MOSFET on, if the supply voltage

is exceeding the over voltage protection level VOV(OFF). The IC operates in normal mode again with a hysteresis VOV(HY) if the supply voltage decreases below the switch-on voltage VOV(ON). In H-bridge configuration, this behavior of the BTS 7960 will lead to freewheeling in highside during over voltage.

### Undervoltage Shut Down

To avoid uncontrolled motion of the driven motor at low voltages the device shuts off (output is tri-state), if the supply voltage drops below the switch-off voltage VUV(OFF). The IC becomes active again with a hysteresis VUV(HY) if the supply voltage rises above the switch-on voltage VUV(ON).

### Overtemperature Protection

The BTS 7960 is protected against overtemperature by an integrated temperature sensor. Overtemperature leads to a shut down of both output stages. This state is latched until the device is reset by a low signal with a minimum length of treset at the INH pin, provided that its temperature has decreased at least the thermal hysteresis T in the meantime.

Repetitive use of the overtemperature protection might reduce lifetime.

### Current Limitation

The current in the bridge is measured in both switches. As soon as the current in forward direction in one switch (high side or low side) is reaching the limit ICLx, this switch is deactivated and the other switch is activated for tCLS . During that time all changes at the

60

40	40

35	35

0	500	1000	1500	2000	0	500	1000	1500	2000

dIL/dt [A/ms]	dIL/dt [A/ms]

60

55

ICLH0

55

50

50

I

CLL0

45

45

Tj = -40°C Tj = 25°C

Tj = 150°C

65

Tj = 150°C

65

70

70

Tj = 25°C

75

Tj = -40°C

75

80

80

Low SideSwitch

High Side Switch

IN pin are ignored. However, the INH pin can still be used to switch both MOSFETs off. After tCLS the switches return to their initial setting. The error signal at the IS pin is reset after 2 * tCLS. Unintentional triggering of the current limitation by short current spikes (e.g. inflicted by EMI coming from the motor) is suppressed by internal filter circuitry. Due to thresholds and reaction delay times of the filter circuitry the effective current limitation level ICLx depends on the slew rate of the load current dI/dt as shown in Figure 8

t

tCLS

IL

ICLx ICLx 0

Figure 7    Timing Diagram Current Limitation

ICL H

[A]

IC LL

[A]

Figure 8    Current Limitation Level vs. Current Slew Rate dI/dt

50

18 V 20

Tj = -40°C

4	6	8	10	12	14	16	18 V 20

VS

45

45

Tj = 25°C

40

40

Tj = 150°C

35

Low Side Switch

High Side Switch

35

16

VS

4	6	8	10	12	14

50

55

ICLL

55

ICLH

60

Tj = -40°C Tj = 25°C

Tj = 150°C

60

65

A

65

A

Figure 9    Typical Current Limitation Detection Levels vs. Supply Voltage

In combination with a typical inductive load, such as a motor, this results in a switched mode current limitation. That way of limiting the current has the advantage that the power dissipation in the BTS 7960 is much smaller than by driving the MOSFETs in linear mode. Therefore it is possible to use the current limitation for a short time without exceeding the maximum allowed junction temperature (e.g. for limiting the inrush current during motor start up). However, the regular use of the current limitation is allowed as long as the specified maximum junction temperature is not exceeded. Exceeding this temperature can reduce the lifetime of the device.

### Short Circuit Protection

The device is short circuit protected against

      - output short circuit to ground

      - output short circuit to supply voltage

      - short circuit of load

The short circuit protection is realized by the previously described current limitation in combination with the over-temperature shut down of the device

### Electrical Characteristics - Protection Functions

– 40 C < Tj < 150 C; 8 V < VS < 18 V (unless otherwise specified)

| Pos. | Parameter | Symbol | Limit Values |  |  | Unit | Test Conditions |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | min. | typ. | max. |  |  |

Under Voltage Shut Down

| 4.3.1 | Switch-ON voltage | VUV(ON) | – | – | 5.5 | V | VS increasing |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 4.3.2 | Switch-OFF voltage | VUV(OFF) | 4.0 | – | 5.4 | V | VS decreasing |
| 4.3.3 | ON/OFF hysteresis | VUV(HY) | – | 0.2 | – | V | – |

Over Voltage Lock Out

| 4.3.4 | Switch-ON voltage | VOV(ON) | 27.5 | – | – | V | VS decreasing |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 4.3.5 | Switch-OFF voltage | VOV(OFF) | 27.6 | – | 30 | V | VS increasing |
| 4.3.6 | ON/OFF hysteresis | VOV(HY) | – | 0.2 | – | V | – |

Current Limitation

| 4.3.7 | Current limitation detection level high side | ICLH0 | 47 44 43 | 62 60 59 | 84 80 79 | A | VS=13.5 V Tj = -40 C Tj = 25 C Tj = 150 C |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 4.3.8 | Current limitation detection level low | ICLL0 | 36 | 47 | 64 | A | VS=13.5V Tj = -40 C |
|  | side |  | 34 | 43 | 61 |  | Tj = 25 C |
|  |  |  | 33 | 42 | 61 |  | Tj = 150 C |

Current Limitation Timing

| 4.3.9 | Shut off time for HS and LS | tCLS | 70 | 115 | 210 | µs | VS=13.5V |
| --- | --- | --- | --- | --- | --- | --- | --- |

Thermal Shut Down

| 4.3.10 | Thermal shut down junction temperature | TjSD | 152 | 175 | 200 | C | – |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 4.3.11 | Thermal switch on junction temperature | TjSO | 150 | – | 190 | C | – |
| 4.3.12 | Thermal hysteresis | T | – | 7 | – | K | – |
| 4.3.13 | Reset pulse at INH pin (INH low) | treset | 3 | – | – | µs | – |

### Control and Diagnostics

### Input Circuit

The control inputs IN and INH consist of TTL/CMOS compatible schmitt triggers with hysteresis which control the integrated gate drivers for the MOSFETs. Setting the INH pin to high enables the device. In this condition one of the two power switches is switched on depending on the status of the IN pin. To deactivate both switches, the INH pin has to be set to low. No external driver is needed. The BTS 7960 can be interfaced directly to a microcontroller.

### Dead Time Generation

In bridge applications it has to be assured that the highside and lowside MOSFET are not conducting at the same time, connecting directly the battery voltage to GND. This is assured by a circuit in the driver IC, generating a so called dead time between switching off one MOSFET and switching on the other. The dead time generated in the driver IC is automatically adjusted to the selected slew rate.

### Adjustable Slew Rate

In order to optimize electromagnetic emission, the switching speed of the MOSFETs is adjustable by an external resistor. The slew rate pin SR allows the user to optimize the balance between emission and power dissipation within his own application by connecting an external resistor RSR to GND.

### Status Flag Diagnosis With Current Sense Capability

The status pin IS is used as a combined current sense and error flag output. In normal operation (current sense mode), a current source is connected to the status pin, which delivers a current proportional to the forward load current flowing through the active high side switch. If the high side switch is inactive or the current is flowing in the reverse

direction no current will be driven except for a marginal leakage current IIS(LK). The external resistor RIS determines the voltage per output current. E.g. with the nominal value of 8500 for the current sense ratio kILIS = IL / IIS, a resistor value of RIS = 1k leads to VIS = (IL / 8.5 A)V. In case of a fault condition the status output is connected to a current source which is independent of the load current and provides IIS(lim). The maximum voltage at the IS pin is determined by the choice of the external resistor and the supply voltage. In case of current limitation the IIS(lim) is activated for 2 * tCLS.

Normal operation:	Fault condition:

current sense mode	error flag mode VS		VS

ESD-ZD	ESD-ZD

IS	IS

IIS~ ILoad

I

Sense output logic

RIS

VIS

Sense output logic

RIS

VIS

IS(lim)

I

IS(lim)

Figure 10	Sense current and fault current

### Truth Table

| Device State | Inputs |  | Outputs |  |  | Mode |
| --- | --- | --- | --- | --- | --- | --- |
|  | INH | IN | HSS | LSS | IS |  |
| Normal operation | 0 | X | OFF | OFF | 0 | Stand-by mode |
|  | 1 | 0 | OFF | ON | 0 | LSS active |
|  | 1 | 1 | ON | OFF | CS | HSS active |
| Over-voltage (OV) | X | X | ON | OFF | 1 | Shut-down of LSS, HSS activated, error detected |
| Under-voltage (UV) | X | X | OFF | OFF | 0 | UV lockout |
| Overtemperature or short circuit of HSS or LSS | 0 | X | OFF | OFF | 0 | Stand-by mode, reset of latch |
|  | 1 | X | OFF | OFF | 1 | Shut-down with latch, error detected |
| Current limitation mode | 1 | 1 | OFF | ON | 1 | Switched mode, error detected |
|  | 1 | 0 | ON | OFF | 1 | Switched mode, error detected |

| Inputs: | Switches | Status Flag IS: |
| --- | --- | --- |
| 0 = Logic LOW | OFF = switched off | CS = Current sense mode |
| 1 = Logic HIGH | ON = switched on | 1 = Logic HIGH (error) |
| X = 0 or 1 |  |  |

Block Description and Characteristics

### Electrical Characteristics - Control and Diagnostics

– 40 C < Tj < 150 C, 8 V < VS < 18 V (unless otherwise specified)

| Pos. | Parameter | Symbol | Limit Values |  |  | Unit | Test Conditions |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | min. | typ. | max. |  |  |
| Control Inputs (IN and INH) |  |  |  |  |  |  |  |
| 4.4.1 | High level voltage INH, IN | VINH(H) VIN(H) | – | 1.75 1.6 | 2.15 2 | V | – |
| 4.4.2 | Low level voltage INH, IN | VINH(L) VIN(L) | 1.1 | 1.4 | – | V | – |
| 4.4.3 | Input voltage hysteresis | VINHHY VINHY | – – | 350 200 | – – | mV | – |
| 4.4.4 | Input current | IINH(H) IIN(H) | – | 30 | 150 | µA | VIN = VINH= 5.3 V |
| 4.4.5 | Input current | IINH(L) IIN(L) | – | 25 | 125 | µA | VIN = VINH=0.4 V |

Current Sense

| 4.4.6 | Current sense ratio in static on-condition kILIS = IL / IIS | kILIS | 6 5 3 | 8.5 8.5 8.5 | 11 12 14 | 103 | RIS = 1 k IL = 30 A IL = 15 A IL = 5 A |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 4.4.7 | Maximum analog sense current, sense current in fault condition | IIS(lim) | 4 | 4.5 | 7 | mA | VS = 13.5 V RIS = 1k |
| 4.4.8 | Isense leakage current | IISL | – | – | 1 | µA | VIN= 0 V or VINH= 0 V |
| 4.4.9 | Isense leakage current, active high side switch | IISH | – | 1 | 200 | µA | VIN = VINH= 5 V IL= 0 A |

Thermal Characteristics

# Thermal Characteristics

| Pos | Parameter | Symbol | Limits |  | Unit | Test Condition |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | min | max |  |  |
| 5.0.1 | Thermal Resistance Junction-Case, Low Side Switch Rthjc(LS) = Tj(LS)/ Pv(LS) | Rthjc(LS) | – | 1.8 | K/W |  |
| 5.0.2 | Thermal Resistance Junction-Case, High Side Switch Rthjc(HS) = Tj(HS)/ Pv(HS) | Rthjc(HS) | – | 0.9 | K/W |  |
| 5.0.3 | Thermal Resistance Junction-Case, both Switches Rthjc= max[Tj(HS), Tj(LS)] / (Pv(HS) + Pv(LS)) | Rthjc | – | 1.0 | K/W |  |
| 5.0.4 | Thermal Resistance Junction-Ambient | Rthja | – | 35 | K/W | 6cm2 cooling area |

Application

# Application

SR

SR

IS

OUT

OUT

IS

IN

M

IN

INH	INH

VS

High Current H-Bridge

VS

µC

GND	GND

BTS 7960B

BTS 7960B

SPD 50P03L

VS

Reverse Polarity Protection

Voltage Regulator

Microcontroller

GND

I

TLE 4278G

WO RO Q

D

Vss

I/O I/O I/O I/O

I/O

Reset Vdd

Application Example

Figure 11	Application Example: H-Bridge with two BTS 7960B

### Layout Considerations

Due to the fast switching times for high currents, special care has to be taken to the PCB layout. Stray inductances have to be minimized in the power bridge design as it is necessary in all switched high power bridges. The BTS 7960 has no separate pin for power ground and logic ground. Therefore it is recommended to assure that the offset between the ground connection of the slew rate resistor, the current sense resistor and ground pin of the device (GND / pin 1) is minimized. If the BTS 7960 is used in a H-bridge or B6 bridge design, the voltage offset between the GND pins of the different devices should be small as well.

A ceramic capacitor from VS to GND close to each device is recommended to provide current for the switching phase via a low inductance path and therefore reducing noise and ground bounce. A reasonable value for this capacitor would be about 470 nF.

The digital inputs need to be protected from excess currents (e.g. caused by induced voltage spikes) by series resistors in the range of 10 k.

Package Outlines P-TO-263-7

# Package Outlines P-TO-263-7

P-TO-263-7 (Plastic Transistor Single Outline Package)

9.9

7.5

6.6

A

1.3 +0.1

-0.02

B

1)

0.1

17

0...0.15

2.4

7 x 0.6 +0.1

-0.03

6 x

AB

0.25 M

B

0.1

0.05

1.27

(14.9)

10.2 ±0.15

9 .2 ±0.2

1 ±0.3

6.5

4 .7 ±0.5

2.7 ±0.5

4.4

0.5

±0.15

8° MAX.

1) Shear and punch direction no burrs this surface Back side, heatsink contour

All metal sufaces tin plated, except area of cut .

0.47

0.8

8.42

16.15	 4.6

9.4

Footprint	10.8

HLGF1019

You can find all of our packages, sorts of packing and others in our Infineon Internet Page “Products”: http://www.infineon.com/products.

SMD = Surface Mounted Device

Dimensions in mm

17.5 ±0.3

15.6 ±0.3

13

3.7 -0.15

8.6 ±0.3

10.2 ±0.3

2.8 ±0.2

3.3 ±0.3

.2 ±0.2

Package Outlines P-TO-220-7

# Package Outlines P-TO-220-7

AB

9.9±0.2

A

9.5±0.2

4.4

7.5

6.6

2.4

0.05

0.25 M

C

1.3+0.1

4.5±0.3

8.4±0.3

7 x 0.6±0.1

6 x 1.27

0.5±0.1

C	17

0...0.15

1)

B

-0.0 2

P-TO-220-7 (Plastic Transistor Single Outline Package)

1) Shear and punch direction no burrs this surface Back side, heatsink contour

All metal surfaces tin plated, except area of cut.

You can find all of our packages, sorts of packing and others in our Infineon Internet Page “Products”: http://www.infineon.com/products.

Dimensions in mm

Revision History

# Revision History

| Revision | Date | Changes / Comments |
| --- | --- | --- |
| n.a. | 2004-03-18 | Target Data Sheet |
| 0.9 | 2004-10-10 | Target Data Sheet converted to new layout |
| 1.0 | 2004-11-30 | Preliminary Data Sheet |
| 1.1 | 2004-12-07 | “Preliminary” removed; No other changes |

Edition 2004-12-07

Published by Infineon Technologies AG, St.-Martin-Strasse 53,

D-81541 München, Germany

© Infineon Technologies AG 2004-12-07. All Rights Reserved.

Attention please!

The information herein is given to describe certain components and shall not be considered as warranted characteristics.

Terms of delivery and rights to technical change reserved.

We hereby disclaim any and all warranties, including but not limited to warranties of non-infringement, regarding circuits, descriptions and charts stated herein.

Infineon Technologies is an approved CECC manufacturer.

Information

For further information on technology, delivery terms and conditions and prices please contact your nearest Infineon Technologies Office in Germany or our Infineon Technologies Representatives worldwide.

Warnings

Due to technical requirements components may contain dangerous substances. For information on the types in question please contact your nearest Infineon Technologies Office.

Infineon Technologies Components may only be used in life-support devices or systems with the express written approval of Infineon Technologies, if a failure of such components can reasonably be expected to cause the failure of that life-support device or system, or to affect the safety or effectiveness of that device or system. Life support devices or systems are intended to be implanted in the human body, or to support and/or maintain and sustain and/or protect human life. If they fail, it is reasonable to assume that the health of the user or other persons may be endangered.

http: //w ww .in f in eon. com

Published by Infineon Technologies AG